Document (Technical Details for Proposed Robot)

# Type of Robot.

Maze Solver Robot – Autonomous Robot

# Robot Assembly Design (Proposed Diagram): Drawings each part of the robot is preferred as an attachment. (CAD drawings are preferred).

Figure 1.1 Side View	Figure 1.2 Front View

Figure 1.3 Top View	Figure 1.4 Normal View

Figure 1.5 Schematic of Proposed Micromouse

Figure 1.6 PCB Layout

- Components to be used: Enlist all the components with their make/company in four groups as enlisted in the following

- List of Structure components: like beams, bushes, shafts, belts, plates, pins, pullies, wheel, connectors, batteries, motors etc.

| COMPONENT | DESCRIPTION | VISUAL REPRESENTATION |
| --- | --- | --- |
| Encoder Motors | - The encoder motor is essential for giving the bot exact control over its movement and speed. - It aids in PID alignment and the bot's turning. |  |
| Li-Po Battery | - Batteries are the main electrical power source for the robot that solves mazes. - It powers the robot's motors, microcontrollers, sensors, and other electronic parts. |  |
| N20 Wheels | - The robot that solves mazes uses the N20 wheels as its propulsion mechanism. - The robot's path through the labyrinth. |  |

- List of Motion Components: like chain, sprockets, flaps etc.

| COMPONENT | DESCRIPTION | VISUAL REPRESENTATION |
| --- | --- | --- |
| Motor socket | - The motor socket is used to hold the motor in a particular position. - It also ensures the safety of the motor and motor connection. |  |
| N20 caster wheel | - Caster wheels are typically placed at the front or rear of a robot to provide pivoting support. |  |

- List of electronics components like smart pods, switches, joysticks, controllers, LED/LCD screen, power supply, programming components etc.

| COMPONENT NAME | ROLE | VISUAL REPRESENTATION |
| --- | --- | --- |
| STM 32 | - The maze-solving robot's brain is housed in an STM32 microprocessor. - It records and optimizes the path, combines sensor data, runs the maze-solving algorithm, regulates motor movements, maintains communication, effectively manages power, and handles faults. |  |
| ST-LINK | - The hardware part called ST LINK is utilized for debugging and programming the Microcontroller STM32. |  |
| TOF (Time of Flight) Sensor | - The Time-of-Flight Sensor used to find the distance between the robot and wall by calculating the time taken for the light to travel. - With great accuracy and wide range, it will find the distance of the wall. |  |
| Pololu Motor Driver | - The motor driver is used to input supply voltage for the encoder motors. - It is used to control the speed and direction of robot. |  |

- List of other Accessories: clothes, plastic eyes/ear/feeling like real all external components which are for the look.

| COMPONENT NAME | ROLE | VISUAL REPRESENTATION |
| --- | --- | --- |
| Soldering station | - Soldering station is used to fix all the components in the printed circuit board with the help of soldering lead. |  |
| Soldering lead | - Soldering lead used to solider the components in the board. |  |
| PCB (Printed Circuit Board) | - The Printed Circuit Board act as a base of the maze solver robot. - It provides solid Mechanical base for mounting and supporting of the electronics components like STM32, Motor driver, encoder motors, sensors. |  |

- The methodology of Making Robot: Please write technical specifications of proposed Robot with brief notes and diagrams

Micromouse is a wheeled robot that solves the unknown maze. The mouse must keep track of its location, find walls as it explores, map out the maze, and recognize when it has arrived at the desired destination. Micromouse can automatically memorize maze data, choose the shortest way, and then accelerate to the predetermined destination. The main scope of this project is to build an STM32 microcontroller-based autonomous maze solver micromouse. Micromouse is made of STM32F103C8T6, TB6612FNG motor driver, N20 motors, and VL61806TOF distance ranging sensor. We had interfaced STM32 with five TOF Sensors, TB6612FNG Motor driver and two motors. The main principle of the micro mouse is to solve the maze and find its destination as soon as possible. PID and flood fill algorithms are used with this mouse. A Double-layer printed circuit board (PCB) micromouse has been designed. It will be fabricated and assembled at an in-house facility. The following algorithms are used to find the shortest path to reach the destination.

# Wall Follow:

The wall-following algorithm uses the existence of walls to guide a robot through a maze. Except for the maze's boundaries, the robot initially expects no walls. The robot recognizes walls to its right, left, and front as it goes. The robot's primary goal is to follow the wall to its right. It consistently makes the right turn when it is possible, ensuring that it remains adjacent to the wall. It goes around the outside of the maze to the left when it comes to a corner or dead end.

# Flood Fill Algorithm:

The flood fill algorithm mimics how water moves through a maze in order to function. Cells in a Flood array are first given values, with the target cells receiving a value of zero and the farther away cells receiving progressively higher values. After being positioned inside the maze, the robot uses the Flood array to identify the values of the cells nearby and then goes to the one with the lowest value. It updates the maze array in accordance with the walls it detects to the left, right, and front. The robot moves in the same direction toward cells with decreasing Flood array values until it reaches the target position. This operation is repeated.

Update Wall

Flood Maze

Analysis the next

shortest path

Figure 4.2.3 Method of Solving

Figure 4.2.2 Flood Fill

Figure 4.2.1 Flow of Flood Fill Algorithm

# PID (Proportional Integral Derivative):

A PID controller, which stands for Proportional-Integral-Derivative controller, is a widely used feedback control mechanism in engineering and automation. It aims to regulate a system's behavior by continuously adjusting an actuator (such as a valve or motor) based on the difference between a desired set point and the measured process variable

# Application of proposed Robot in a societal context: Not more than 100 words.

The importance and versatility of micro-mouses are demonstrated by their everyday applications. The ideas of autonomous navigation and efficient problem-solving pioneered in micro- mouse research have a wide range of practical applications, from improving healthcare and logistics to home automation and personal convenience.

- Automated medical dispenser: Micro mouse can be utilized in hospitals to provide patients with medication automatically over a predetermined length of time.

- Autonomous Guide: In shopping centres, supermarkets, shops, and factories, the micro mouse also serves as a guide. The consumer will find the path (shortest path) with its assistance.

# Size of Robot proposed for Proof of Concept (Small Version):

- Length in cm - 8.2 cm

- Width in cm - 8 cm

- Height in cm – 5 cm

# Size of Robot proposed as prototype (Actual Version):

- Length in cm - 8.2 cm

- Width in cm - 8 cm

- Height in cm - 5 cm

# Timeline for Robot Making with milestones. (Divided in activities Vs. no. of days)

| S. No | Area of Work | Days Required |
| --- | --- | --- |
| 1. | Research | 2 days |
| 2. | PCB design and Fabrication | 8 days |
| 3. | Coding and Testing | 10 days |
| 4. | Component Assembly and Testing | 8 days |
| 5. | PID Implementation | 3 days |
| 6. | Overall Testing | 2 days |

- Please attach the proposed outline (photography) for the understanding of the evaluation committee.

Figure 9.1 Proposed Block Diagram

Figure 9.2 Right Way of solving the maze	Figure 9.3 Goal of the Maze

Figure 9.4 The Way of Thinking	Figure 9.5 Shortpath Detection